Files for the PCB

=================

Gerbers (copper layers, in order, top to bottom)

spectranet.front.gbr  - Component side

spectranet.group2.gbr - Layer 2

spectranet.group4.gbr - Layer 3

spectranet.back.gbr - Solder side

Solder mask and silkscreen Gerbers:

spectranet.frontsilk.gbr  - Component side silk screen

spectranet.frontmask.gbr  - Component side solder mask

spectranet.backmask.gbr   - Solder side solder mask

Other Gerbers

spectranet.outline.gbr    - Board outline (includes cutout for slot)

Drill file:

spectranet.plated-drill.cnc - All holes are through plated.

Assembly request

================

I would like component sourcing and assembly for the components listed in

the BOM. The files for the BOM are as follows:

spectranet-bom.xls  - BOM file as your BOM template

spectranet.bom    - BOM file as output by gEDA PCB (plain text)

spectranet.xy   - XY BOM showing placement, as output by gEDA PCB

XY positions are in inches. The file is plain text.

Since I am not sure if you can make use of the xyfile there are two PNG

images to help locate the components:

spectranet-assy.png - Close-up of the cluttered area in the top left

hand of the PCB, near the W5100. I have put the

refdes of the components in red where it may be

difficult to tell where the component should be

soldered.

spectranet.png    - A picture of the component side of the board

as generated by gEDA PCB

spectranet-photo.jpg  - A photograph of the previous revision of the board

which I hand assembled (the only difference in

assembly is the addition of R17 near the LINK LED)

One thing of note is the IDT71024 static RAM IC has a "half moon" on its IC

package, but this does not indicate which end has pin 1 (I have 71024s with

the half moon at the pin 1 end, and some with the half moon at the pin 16

end). The only indicator of pin 1 for this IC is the small hole beside pin 1.

This can be seen in the photograph. Also the orientation of the text on the

IC package.

The Wiznet W5100 IC is not available from Farnell, but can be ordered

directly from the Wiznet website. Everything else in the BOM has a

Farnell part number.

http://www.wiznet.co.kr/en/

Note that I will assemble the card edge connector and the through hole

components myself; I only need SMD components to be assembled. There is

a paste mask Gerber if it will be useful:

spectranet.frontpaste.gbr - Solder paste file